| Code | Error Messages | Error | Measures |
| --- | --- | --- | --- |
| E-00 | ERROR 00 | No error occurs. | This is not usually displayed. |
| E-01 | ERROR 01 | Either motor of main shaft, X- or Y-axis, or lower shaft has locked. |  |
| E-02 | Overtravel | Overtravel occurs during home position detecting movement. |  |
| E-03 | Stop SW was pressed during home positioning | The stop switch is pressed during home position detecting movement. | Press the or turn on the F/B switch on the head to either side to restart the home position detecting movement again. |
| E-04 | Home position detection is out of range | Home position detection out of range | This is not usually displayed. |
| E-05 | Needle stop position error | Needle stop position error | Adjust the pulley stop position (100 degrees) above the needle and press the or turn on the F/B switch on the head to either side. |
| E-06 | Needle bar case position error | Needle bar case position error | Adjust the position of needle bar case manually, then press the or turn ON the F/B switch on the head to either side. If the same error occurs again, the color change proper position sensor is faulty. |
| E-07 | Needle case lock | Needle bar case lock | Press the or turn on the F/B switch on the head to either side. If the same error occurs again, the color change mechanism is faulty. |
| E-08 | ERROR 08 | Stop switch or emergency switch was pressed while the needle bar case is traveling. | This is not usually displayed. |
| E-09 | X-axis home position error | X-axis home position detection error | Turn the power off and on once. If the same error occurs again, the X-axis mechanism is faulty. |
| E-0A | Thread breakage error | Thread breaking error | After passing through the thread, press the or turn on the F/B switch on the head to either side. |
| E-0B | ERROR 0B | Stop or emergency stop during sewing | This is not usually displayed. |
| E-0C | Lower thread breakage error | Lower thread broke during embroidering. | After the replacement of the lower thread, press the or turn ON the F/B switch on the head to either side. |
| E-0D | ERROR 0D | The machine does not return to the home position. | This is not usually displayed. |
| E-0E | ERROR 0E | Mending finish |  |
| E-0F | Needle bar No. reading error | Failed in the reading of needle bar No. | Press the or turn ON the F/B switch on the head to either side. If the same error occurs again, the I/O CPU communication is faulty, or the color change position sensor is faulty. |
| E-10 | The beam sensor turned ON | Abort by the beam sensor ON | Press the or turn ON the F/B switch on the head to either side. |
| E-11 | X sensor reading error | Failed in the reading of X home position sensor | Press the or turn ON the F/B switch on the head to either side. If the same error occurs again, the I/O CPU communication is faulty. |
| E-12 | Y sensor reading error | Failed in the reading of X home position sensor | Press the or turn ON the F/B switch on the head to either side. If the same error occurs again, the I/O CPU communication is faulty. |
| E-13 | Performing the home position detection for the hoop position adjustment | This is displayed in case that the machine had been shut down while the hoop was moving and now is started up in a mode without the home position detection, or in case that the machine is started up in a mode without the home position detection after pressing the CAP SW to change the hoop. | Press the to start the home position moving. |
| E-14 | Y-axis home position error | Y-axis home position error | Turn the power off and on once. If the same error occurs again, the Y-axis mechanism is faulty. |
| E-15 | Press for restart. | Stop error during SSP processing (when pressing the stop key while the hoop is moving) | Hoop movement restarts if you press. |
| E-16 | Needle bar No. of the destination is abnormal | Needle bar No. of the destination is out of range | Press the , or turn ON the F/B switch on the head to either side to cancel the error and set up the needle bar setting again. |
| E-17 | Speed range abnormal | Speed range is out of range | This is not usually displayed. |
| E-18 | X-axis stepping motor connector error | X-axis stepping motor connector error | Turn the power off and on once after checking to see that the connector of the X-axis stepping motor is properly connected. |
| E-1A | ERROR 1A | Destination coordinates error | This is not usually displayed. |
| E-1B | ERROR 1B | The machine has reached the mending stop position. |  |
| E-1C | Restart perimeter | The machine stops during mask tracing. | Tracing is cancelled if the key is pressed when the machine is stopped during mask tracing. Press the key to continue tracing. |
| E-1D | Stop while transferring to next repeat pattern | The machine stops while the needle is moving between patterns during repeat sewing. | This is displayed when the stop switch is pressed while the hoop is moving. Press the to move the hoop again. (It is necessary to press the again to start sewing.) |
| Errors E-1C and E-1D are not displayed due to mechanical problems. |  |  |  |
| E-1E | Remove unused presser foot, or it may be damaged | When the power is turned on, bed can not be retracted. | This is not displayed in this model. |
| E-1F | Presser foot down error | Presser foot down error while searching for home position just after the power is turned on. |  |
| E-21 | Area over | Hoop overhang (+X) | Pattern or the needle position is out of the embroidering area. Reset the embroidering area on the panel or move the hoop to the sewable position. |
| E-22 | Area over | Hoop overhang (+Y) |  |
| E-23 | Area over | Hoop overhang (+X, +Y) |  |
| E-24 | Area over | Hoop overhang (-X) |  |
| E-25 | Area over | Hoop overhang (+X, -X) |  |
| E-26 | Area over | Hoop overhang (-X, +Y) |  |
| E-27 | Area over | Hoop overhang (+X, -X, +Y) |  |
| E-28 | Area over | Hoop overhang (-Y) |  |
| E-29 | Area over | Hoop overhang (+X, -Y) |  |
| E-2A | Area over | Hoop overhang (+Y, -Y) | Pattern or the needle position is out of the embroidering area. Reset the embroidering area on the panel or move the hoop to the sewable position. |
| E-2B | Area over | Hoop overhang (+X, +Y, -Y) |  |
| E-2C | Area over | Hoop overhang (-X, -Y) |  |
| E-2D | Area over | Hoop overhang (+X, -X, -Y) |  |
| E-2E | Area over | Hoop overhang (-X, +Y, -Y) |  |
| E-2F | Area over | Hoop overhang (+X, -X, +Y, -Y) |  |
| E-31 | Area over | Needle overhang (+X) |  |
| E-32 | Area over | Needle overhang (+Y) |  |
| E-33 | Area over | Needle overhang (+X, +Y) |  |
| E-34 | Area over | Needle overhang (-X) |  |
| E-36 | Area over | Needle overhang (-X, +Y) |  |
| E-38 | Area over | Needle overhang (-Y) |  |
| E-39 | Area over | Needle overhang (+X, -Y) |  |
| E-3C | Area over | Needle overhang (-X, -Y) |  |
| E-40 | Remove unused presser foot, or it may be damaged | The status of presser foot is issued as an alarm when every second machine is used. | This is not displayed in this model. |
| E-41 | This function is not supported. Turn off the power | Invalid start-up error |  |
| E-42 | XY movement error | XY movement error | Press the or turn on the F/B switch on the head to either side. If this error occurs frequently, check if the tension of the pulley belt is proper. |
| E-43 | Slave I/F EEPROM read error | I/F CPU failed to read EEPROM data | Turn the power off and on once. If the same error occurs again, the main PC is faulty. |
| E-44 | Inter-head CPU communication error | Failed in the communication with inter-head CPU | Press the or turn ON the F/B switch on the head to either side. If the same error occurs again, the connector of communication cable on the inter-head CPU may be disconnected. |
| E-45 | Head CPU communication error | Failed in the communication with head CPU | Press the or turn ON the F/B switch on the head to either side. If the same error occurs again, the connector of communication cable on the head CPU may be disconnected. |
| E-46 | XY CPU communication error | Failed in the communication with XY CPU | Press the or turn ON the F/B switch on the head to either side. If the same error occurs again, the main PCB is faulty. |
| E-47 | I/F CPU communication error | Failed in the communication with I/F CPU | Press the or turn ON the F/B switch on the head to either side. If the same error occurs again, the main PCB is faulty. |
| E-48 | Main shaft rotation abnormal | The main shaft rotated the opposite way | Turn OFF the power and check if the sensors of the encoder A and B are connected correctly, not mixed-up each other. If those connections are correct, then check if the pin connections of the main shaft motor connectors are correct. |
| E-49 | Can not identify the machine model | Can not identify the machine model | Turn OFF the power, check if the model identification harness is connected properly, and turn ON the power again. |
| E-4A | I/O CPU communication error | Failed in the communication with I/O CPU | Press the or turn ON the F/B switch on the head to either side. If the same error occurs again, the connector of communication cable on the I/O PCB may be disconnected. |
| E-A1 | Main (Z) motor lock | Spindle motor lock | Press the or turn on the F/B switch on the head to either side. If it occurs frequently, the main shaft mechanism is faulty. |
| E-A2 | Main (Z) PCB temperature is too high | Main PC board temperature too high | This is not usually displayed. |
| E-A3 | Main (Z) motor voltage is too low | Spindle motor voltage too low |  |
| E-A4 | Main (Z) motor voltage is too high | Spindle motor voltage too high |  |
| E-A5 | Spindle CPU error | Spindle motor CPU error |  |
| E-A6 | ERROR A6 | Spindle motor CPU communication command error |  |
| E-A7 | ERROR A7 | Spindle motor CPU send/receive error |  |
| E-A8 | ERROR A8 | Spindle stop position signal error | Adjust the pulley stop position (100 °) above the needle and press the . If the error occurs frequently, the parts related to the main shaft stop position sensor are faulty. |
| E-A9 | Spindle CPU parameter error | Spindle CPU parameter error | This is not displayed in this model. |
| E-B0 | Lower shaft CPU error | Lower shaft CPU error |  |
| E-B1 | Thread jammed in rotary hook | Thread tangle in rotary hook |  |
| E-B2 | Hook motor origin point error | Hook motor origin point error |  |
| E-B3 | Hook motor standby position error | Hook motor standby position error |  |
| E-B4 | Hook motor standby position error | Hook motor motor mode error |  |
| E-B5 | Hook motor communication error | Hook motor communication error |  |
| E-B6 | Hook motor parameter error | Hook motor parameter error |  |
| E-B7 | Hook motor overheat error | Hook motor overheat error |  |
| E-B8 | Hook motor overcurrent error Turn off the power | Hook motor overcurrent error |  |
| E-B9 | Thread trimming motor origin point error | Thread trimming motor zero point error | Turn the power off and on once. If the same error occurs again, the lower shaft motor is faulty. |
| E-BA | Power supply frequency error | Power supply frequency error | Turn the power off and check the thread trimmer and turn the power on again. If it occurs again, the thread trimmer is faulty. |
| E-BC | No power error | No power error | It may usually occur momentarily when turning off the machine. If it occurs when the machine is on, it is a power failure detection error. |
| E-BD | Lower shaft flash memory error | Lower shaft flash memory error | This is not displayed in this model. |
| E-BE | Lower shaft version-up error | Lower shaft version-up error |  |
| E-BB, BF | ERROR BB, BF | Lower shaft motor undefined error |  |
| E-C1 | ERROR C1 | Area over during embroidering |  |
| E-C2 | Wiper out error | Wiper out error | If the wiper is tangled with a thread, remove it. Press the or turn on the F/B switch on the head to either side. |
| E-C3 | ERROR C3 | Embroidering data buffer empty | Check the connection of the communication cable of the panel. If the cable is connected correctly , press the or turn on the F/B switch on the head to either side. If the cable is disconnected , turn off the power and connect the cable. |
| E-C4 | Presser foot down error | Presser foot down error | This is not usually displayed. |
| E-C5 | ERROR C5 | Measured voltage value could not be received form the lower shaft motor CPU. |  |
| E-C6 | ERROR C6 | Voltage value could not be transferred to the lower shaft motor CPU. |  |
| E-C7 | ERROR C7 | Lower shaft error |  |
| E-C8 | ERROR C8 | Hook motor origin point error |  |
| E-C9 | Embroidering start error | Embroidering start error |  |
| E-CA | ERROR CA | No sewing permisson |  |
| E-CB | Spindle rotation speed error | Spindle rotation speed error | Press the or turn on the F/B switch on the head to either side to cancel the error and press the . If the same error occurs again, there is a possibility that the spindle is overloaded. |
| E-CC | ERROR CC | Shuttle thread entanglement automatic reset | This is not usually displayed. |
| E-CD | Spindle rotation command sending error | Speed command can not be received. | Turn the power off and on once. If the same error occurs again, the main PCB is faulty. |
| E-CE | Cylinder bed position error | Cylinder bed position error | This is not usually displayed. |
| E-CF | ERROR CF | Rated voltage value could not be received from the lower shaft motor CPU. |  |
| E-D0 | Power PC board error | Power PC board error | Turn the power off and on once. If the same error occurs again, the power PCB is faulty. |
| E-D1 | Cooling fan motor stop | Cooling fan motor stop. | This is not usually displayed. |
| E-D2 | Power voltage upper limit error | Power voltage upper limit error | (1) The voltage setting is improper. -> Set it again. (2) Press the or turn on the F/B switch on the head to either side. If the same error occurs again, the power PCB or the power supply is faulty. |
| E-D3 | Power voltage lower limit error | Power voltage lower limit error |  |
| E-D4 | Thread trimming motor overcurrent | Thread trimming motor overcurrent | Turn OFF the power. The possible causes for this error are, troubles of the thread trimming motor, troubles of the main PCB, and troubles of the drive PCB. |
| E-D5 | Needle bar case motor overcurrent | Needle bar case motor overcurrent | Turn OFF the power. The possible causes for this error are, troubles of the needle bar case motor, troubles of the main PCB, and troubles of the drive PCB. |
| E-D6 | Jump motor overcurrent | Jump motor overcurrent | Turn OFF the power. The possible causes for this error are troubles of the jump motor and troubles of the head PCB. |
| E-D7 | Temperature rise error Clean the fan filter | The temperature of the heat sink on the drive PCB rose excessively | Turn OFF the power and check if the fan filter has any clogging. |
| E-D8 | Temperature sensor abnormal | Temperature sensor abnormal | Turn OFF the power. The possible causes for this error are troubles of the thermistor and troubles of the drive PCB. |
| E-E1 | X-axis pulse motor overcurrent stop Turn off the power | X-axis pulse motor overcurrent stop | Turn OFF the power and ON once. If the same error occurs again, the pulse motor or the drive PCB is faulty. |
| E-E2 | Y-axis pulse motor overcurrent stop Turn off the power | Y-axis pulse motor overcurrent stop |  |
| E-E3 | Exhaust fan motor stop | Cooling fan motor stop A Press R. | Turn OFF the power and check the fan harness. Turn OFF the power again. If the same error occurs again, the fan or the power PCB is faultly. |
| E-E4 | Hook motor error | Lower shaft communication error | This is not displayed in this model. |
| E-E5 | ERROR E5 | Over-run error during interfacing to main PCB CPU | This is not usually displayed. |
| E-E6 | ERROR E6 | Framing error during interfacing to main PCB CPU |  |
| E-E7 | ERROR E7 | Parity error during interfacing to main PCB CPU |  |
| E-E8 | ERROR E8 | Receiving time up error during interfacing to main PCB CPU |  |
| E-E9 | ERROR E9 | Send/Receive inconsistent error during interfacing to main PCB CPU |  |
| E-EA | ERROR EA | ACK code receiving error during interfacing to main PCB CPU |  |
| E-EB | ERROR EB | Send/Receive ID code error during interfacing to main PCB CPU |  |
| E-EC | ERROR EC | Send data checksum error during interfacing to main PCB CPU |  |
| E-ED | ERROR ED | Data empty error during interfacing to main PCB CPU |  |
| E-EE | ERROR EE | Abnormal data received in the inner-machine communication |  |
| E-EF | ERROR EF | Receiving error on interface |  |
| E-F1 | ERROR F1 | Receive time up error |  |
| E-F2 | ERROR F2 | Request-to-waiting time up error |  |
| E-F3 | ERROR F3 | Request-to-receive time up error |  |
| E-F4 | ERROR F4 | Receive command error |  |
| E-F5 | ERROR F5 | NACK code receiving error |  |
| E-F6 | ERROR F6 | Data requested for needle position can not be returned. |  |
| E-F7 | ERROR F7 | It is not receive command for the request one. |  |
| E-F8 | ERROR F8 | PRE code error |  |
| E-F9 | ERROR F9 | No applicable command | This is not usually displayed. |
| E-FA | ERROR FA | Interface receive data check sum error |  |
| E-FB | ERROR FB | Send time up error |  |
| E-FF | ERROR FF | No status is returned from spindle, lower shaft motor, or CPU. |  |